# Frankever Dimmer

DPID (Packet Info):

| First Byte | Second Byte | Third Byte | Fourth Byte | Fifth Byte |
| --- | --- | --- | --- | --- |
| 0xAA (Header) | 0x00 (++ till 0xFF) | - 0x00 (Off) - 0x01 (On) | 0x00 to 0x64 (Value Byte) | Checksum Byte (Add 2nd to 4th Byte values) |

FEATURES:

- Power button to manually On & Off the bulb.

- Increase/Decrease Intensity Button to increase/decrease the intensity of the bulb manually.

- Long press the power button for atleast 20 seconds to run factory reset.

- All above indications are controllable through ezlo cloud.

OBSERVATIONS:

- Bulb is On -> Red LED

- Bulb is Off -> Blue LED

- Wifi Disconnection -> Green LED

Note: Only one indication at a time will be visible.

DAIS:

Devices:

{

"_id": "10364000",

"deviceTypeId": "ezlopi",

"parentDeviceId": "",

"category": "dimmable_light",

"subcategory": "dimmable_bulb",

"gatewayId": "457a5069",

"batteryPowered": false,

"name": "Frankever Dimmer",

"type": "dimmer.outlet",

"reachable": true,

"persistent": false,

"serviceNotification": false,

"armed": false,

"roomId": "",

"security": "no",

"ready": true,

"status": "idle",

"parentRoom": true,

"protectConfig": "default"

}

Items:

{

"_id": "20364000",

"deviceId": "10364000",

"hasGetter": true,

"hasSetter": true,

"name": "dimmer",

"show": true,

"valueType": "int",

"value": 0,

"valueFormatted": "0",

"status": "idle"

},

{

"_id": "20364001",

"deviceId": "10364000",

"hasGetter": true,

"hasSetter": true,

"name": "switch",

"show": true,

"valueType": "bool",

"value": false,

"valueFormatted": "false",

"status": "idle"

}

Pictures:

PCB1 Backside

PCB1 Frontside

PCB2 Backside

PCB2 Frontside

Schematic: